SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE NAME:	CSA07 COURSE CODE: Computer Networks

COURSE OUTCOME COVERED

CO1: Apply the functions of OSI layer in enabling reliable data transmission across networks. (BL2, BL3)

Assessment Tool Weightage: 40%

Annexure A

SHORT ANSWER TEST

Code of Conduct:

I ____________________ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

## Question 1

A company needs to transfer a 150 MB design file from a client computer to a remote server over a network with a bandwidth of 50 Mbps. The Transport Layer divides the file into 1500-byte segments, while the Data Link Layer encapsulates each segment into frames by adding 40 bytes of header and trailer. Calculate the total number of segments and frames transmitted, the total transmission overhead introduced by the Data Link Layer, and estimate the total transmission time (ignoring propagation delay). Explain how the Transport and Data Link layers ensure reliable communication.

Answer:

- Assumptions: 1 MB = 1,000,000 bytes (standard for network transmission calculation).

- Number of Segments and Frames:
File size = 150 MB = 150,000,000 bytes.
Number of Segments = 150,000,000 bytes / 1500 bytes per segment = 100,000 segments.
Since each segment is encapsulated into a single frame, there are 100,000 frames.

- Total Transmission Overhead:
100,000 frames × 40 bytes/frame = 4,000,000 bytes (4 MB).

- Total Transmission Time:
Total data to transmit = Payload + Overhead = 150,000,000 + 4,000,000 = 154,000,000 bytes.
Total bits = 154,000,000 × 8 = 1,232,000,000 bits.
Transmission Time = Total Bits / Bandwidth = 1,232,000,000 bits / 50,000,000 bps = 24.64 seconds.

- Explanation: The Transport Layer ensures reliable communication by segmenting data, maintaining sequence numbers, and using acknowledgments/retransmissions (e.g., TCP) to guarantee end-to-end delivery. The Data Link Layer ensures reliability over the physical medium by adding headers (for physical addressing) and trailers (like the Frame Check Sequence for error detection) to prevent corrupted frames from being processed.

## Question 2

A Transport Layer protocol uses a sliding window size of 6 frames. Each frame carries 1024 bytes of user data. If the sender transmits 48 frames, calculate the number of transmission windows required. If one frame in every window is lost and must be retransmitted, determine the total number of retransmissions. Explain how flow control improves communication efficiency.

Answer:

- Transmission Windows Required:
Total frames / Window size = 48 / 6 = 8 windows.

- Total Number of Retransmissions:
1 frame lost per window × 8 windows = 8 retransmissions.

- Explanation: Flow control ensures that a fast sender does not overwhelm a slow receiver. By negotiating a window size, the sender knows exactly how much data it can transmit before waiting for an acknowledgment. This prevents buffer overflows, minimizes packet loss at the destination, and reduces the need for constant retransmissions, thereby significantly improving overall communication efficiency.

## Question 3

A 12,000-byte IP packet is transmitted through a router where the Maximum Transmission Unit (MTU) is 1500 bytes. Assuming each fragment requires a 20-byte IP header, calculate the total number of fragments created and the total header overhead introduced. Explain how the Network Layer ensures successful delivery of fragmented packets to the destination.

Answer:

- Number of Fragments:
MTU = 1500 bytes. IP Header = 20 bytes. Payload per fragment = 1500 - 20 = 1480 bytes.
Total fragments = 12,000 bytes / 1480 bytes per payload = 8.108.
Since we cannot have a fraction of a fragment, we round up. Total = 9 fragments.
(Note: 8 fragments of 1480 bytes = 11,840 bytes. The 9th fragment will hold the remaining 160 bytes).

- Total Header Overhead:
9 fragments × 20 bytes/header = 180 bytes.

- Explanation: The Network Layer ensures successful delivery of fragmented packets by assigning the same Identification number to all fragments belonging to the original packet. It also uses the Fragment Offset field to indicate the position of the fragment in the original data, and the More Fragments (MF) flag to signal the last fragment. The destination device uses this information to properly reassemble the packet before passing it to the Transport layer.

## Question 4

A Transport Layer protocol uses a sliding window size of 6 frames. Each frame carries 1024 bytes of user data. If the sender transmits 48 frames, calculate the number of transmission windows required. If one frame in every window is lost and must be retransmitted, determine the total number of retransmissions. Explain how flow control improves communication efficiency.

Answer:

- (Note: This question is identical to Question 2 in the provided assignment prompt.)

- Transmission Windows Required: 48 / 6 = 8 windows.

- Total Number of Retransmissions: 8 windows × 1 lost frame = 8 retransmissions.

- Explanation: Flow control manages the rate of data transmission to prevent receiver buffer overflow, optimizing bandwidth usage by avoiding unnecessary packet drops and retransmissions.

## Question 5

A multimedia application transfers a 600 MB video file. The Session Layer inserts a synchronization checkpoint after every 60 MB of transmitted data. During transmission, a failure occurs after 390 MB has been transferred. Determine the number of checkpoints created before failure and calculate the amount of data that must be retransmitted after recovery. Explain the importance of synchronization in the Session Layer.

Answer:

- Checkpoints Created Before Failure:
Amount transferred = 390 MB. Checkpoint interval = 60 MB.
Checkpoints occurred at: 60, 120, 180, 240, 300, and 360 MB.
Number of checkpoints = 390 / 60 = 6.5. Thus, 6 checkpoints were successfully created.

- Data to Retransmit:
The last successful checkpoint was at 360 MB. The failure happened at 390 MB.
Data to retransmit after recovery = Data sent since last checkpoint = 390 MB - 360 MB = 30 MB.

- Explanation: Synchronization in the Session Layer is critical for long data transfers (like large video files). By inserting checkpoints, the session layer allows the system to resume a failed transmission from the last verified checkpoint rather than restarting the entire 600 MB transfer from the beginning. This saves significant bandwidth and time.

## Question 6

Before transmission, the Presentation Layer compresses a 900 MB file by 40%. The compressed data is encrypted, adding 5% overhead to the compressed size. Calculate the compressed file size, the encrypted file size, and the total percentage reduction compared to the original file. Explain how the Presentation Layer improves transmission efficiency and security.

Answer:

- Compressed File Size:
900 MB - (900 × 0.40) = 900 - 360 = 540 MB.

- Encrypted File Size:
540 MB + (540 × 0.05) = 540 + 27 = 567 MB.

- Total Percentage Reduction:
Reduction in size = Original size - Final size = 900 - 567 = 333 MB.
Percentage reduction = (333 / 900) × 100 = 37% reduction.

- Explanation: The Presentation Layer improves transmission efficiency through compression, which drastically reduces the amount of data that needs to be sent over the network, saving bandwidth and lowering transmission time. It improves security through encryption, ensuring that intercepted data cannot be read by unauthorized parties.

## Question 7

An FTP application transfers 3 GB of data over a network with an effective throughput of 120 Mbps. The Application Layer divides the data into requests of 3 MB each. Calculate the total number of requests generated and estimate the total transmission time in seconds. Discuss how the Application Layer provides reliable services to users.

Answer:

- Total Number of Requests:
Total data = 3 GB = 3000 MB (using base-10 conversion typical for network throughput).
Requests = 3000 MB / 3 MB per request = 1000 requests.

- Total Transmission Time:
Total data in bytes = 3,000,000,000 bytes.
Total data in bits = 3,000,000,000 × 8 = 24,000,000,000 bits.
Transmission Time = Total bits / Throughput = 24,000,000,000 bits / 120,000,000 bps = 200 seconds.

- Explanation: The Application Layer provides reliable services by supplying network interfaces directly to user applications (like FTP, HTTP). It implements protocol-specific features (like FTP commands for file integrity checks, resuming downloads, and authentication) that allow users to interact with network resources seamlessly and securely.

## Question 8

A packet experiences the following processing delays at different OSI layers: Application Layer = 4 ms, Transport Layer = 3 ms, Network Layer = 5 ms, Data Link Layer = 2 ms, and Physical Layer = 1 ms. The propagation delay is 18 ms, and the transmission delay is 12 ms. Calculate the total end-to-end delay experienced by the packet and explain how each layer contributes to the communication process.

Answer:

- Total Processing Delay:
4 + 3 + 5 + 2 + 1 = 15 ms.

- Total End-to-End Delay:
Total Delay = Processing Delay + Propagation Delay + Transmission Delay
Total Delay = 15 ms + 18 ms + 12 ms = 45 ms.

- Explanation: Each layer modularizes the communication process:
- Application: Prepares user data for network entry.
- Transport: Segments data and ensures reliability.
- Network: Determines the best path (routing) and logical addressing.
- Data Link: Frames packets for local node-to-node delivery and handles MAC addressing.
- Physical: Converts bits into physical signals (light, electricity) for the physical medium.

## Question 9

A network transfers 80 MB of useful data using frames of 1600 bytes, where each frame contains 80 bytes of protocol overhead added by different OSI layers. Calculate the total number of frames required, the total overhead transmitted, and the transmission efficiency as a percentage. Explain how protocol overhead affects network performance.

Answer:

- Total Number of Frames:
Payload per frame = 1600 bytes - 80 bytes (overhead) = 1520 bytes of useful data.
Total useful data = 80 MB = 80,000,000 bytes.
Frames = 80,000,000 bytes / 1520 bytes = 52,631.57. Rounding up, we need 52,632 frames.

- Total Overhead Transmitted:
52,632 frames × 80 bytes/frame = 4,210,560 bytes (approx 4.21 MB).

- Transmission Efficiency:
Efficiency = (Useful Data per frame / Total Frame Size) × 100 = (1520 / 1600) × 100 = 95%.

- Explanation: Protocol overhead consists of necessary headers/trailers for routing, sequencing, and error checking. While crucial for reliable delivery, high overhead consumes available bandwidth, leaving less room for user data, which effectively lowers network performance and actual throughput.

## Question 10

A hospital transfers a patient's medical record of 240 MB through an IoT-enabled healthcare network. The Presentation Layer compresses the data by 30%, the Transport Layer divides the compressed data into 1400-byte segments, and the Data Link Layer adds 60 bytes of overhead to each frame. Calculate the compressed file size, the total number of segments transmitted, the total protocol overhead introduced, and the final amount of data transmitted across the Physical Layer. Explain how multiple OSI layers work together to ensure secure and reliable communication.

Answer:

- Compressed File Size:
240 MB - (240 × 0.30) = 240 - 72 = 168 MB (168,000,000 bytes).

- Total Number of Segments:
168,000,000 bytes / 1400 bytes per segment = 120,000 segments (and consequently, 120,000 frames).

- Total Protocol Overhead (Data Link):
120,000 frames × 60 bytes/frame = 7,200,000 bytes (7.2 MB).

- Final Amount of Data Transmitted:
Compressed Data + Overhead = 168,000,000 + 7,200,000 = 175,200,000 bytes (175.2 MB).

- Explanation: The OSI layers collaborate to form a robust pipeline. The Presentation Layer shrinks the payload size (improving speed), the Transport Layer chunks the data and guarantees that none of the segments are lost, and the Data Link Layer wraps these segments securely for local transit across switches/routers, detecting any physical errors along the way.

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understanding of Concepts | 25% | Demonstrates a thorough, accurate understanding of concepts | Good understanding with minor gaps | Basic understanding with some misconceptions | Poor understanding; major misconceptions |
| Explanation | 25% | Detailed, well-explained answers with relevant examples | Clear explanation with adequate details | Limited explanation; lacks depth | Poor or unclear explanation |
| Application | 20% | Effectively applies concepts with strong analytical ability | Applies concepts with minor errors | Limited application with weak analysis | No application or incorrect analysis |
| Organization | 15% | Well-structured answers with logical flow and coherence | Organized with minor issues in flow | Basic structure, lacks clarity | Poor organization, incoherent answers |
| Accuracy | 10% | Completely accurate and covers all required points | Mostly accurate with minor omissions | Partially correct with missing points | Incorrect answers with major gaps |
| Clarity | 5% | Clear, neat, professional presentation | Understandable with minor issues | Limited clarity, readability issues | Poorly written, difficult to understand |